SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- DEBUGGING & OPTIMISATION TASK

| Course Code: | CSA1223 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL3- DEBUGGING & OPTIMISATION TASK |
| Weightage: | 20% | Total Marks: | 100 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

- A program performing signed integer addition using 2’s complement produces incorrect results when adding large values (e.g., +120 and +50 in 8-bit representation). Debug the issue by analyzing the binary computation and identifying whether overflow has occurred. Propose a solution to correctly detect and handle overflow in such cases.

ANSWER :

Identification of Error

The program performs signed addition using 8-bit 2’s complement. The given values are:

The addition gives:

01111000

+ 00110010

-----------

10101010

The expected mathematical result is:

But the valid range of an 8-bit signed 2’s-complement number is:

Therefore, +170 is outside the representable range and an overflow has occurred.

Debugging Analysis

Both input numbers are positive because their MSBs are 0.

However, the result is:

10101010

whose MSB is 1, indicating a negative number.

Thus:

Positive + Positive → Negative

This confirms signed overflow.

The 8-bit result 10101010 corresponds to:

Therefore, the root cause is failure to detect signed overflow before interpreting the 8-bit result.

Corrected Solution

Overflow can be detected using the sign-bit condition:

For this example:

A7 = 0

B7 = 0

R7 = 1

Therefore:

The program must set an overflow flag and reject the wrapped result instead of treating 10101010 as the valid answer.

A wider temporary register can also be used:

TEMP = 120 + 50

if TEMP > 127 or TEMP < -128

Overflow = 1

Report overflow

else

Store result

Optimization

To improve the arithmetic implementation:

- Use an overflow flag in the ALU.

- Perform the calculation using a wider temporary register when necessary.

- Prevent the wrapped value from being stored as a valid signed result.

This avoids incorrect results and improves the reliability of the processor's arithmetic unit.

| Test Case | A | B | Expected Result | Overflow |
| --- | --- | --- | --- | --- |
| Test 1 | 120 | 50 | 170 | Detected |
| Test 2 | 100 | 20 | 120 | Not Detected |
| Test 3 | 127 | 1 | 128 | Detected |
| Test 4 | -100 | 50 | -50 | Not Detected |

Result

The program was successfully debugged. The incorrect result occurs because +120 + +50 = +170, which cannot be represented in 8-bit signed 2’s complement. The overflow detection mechanism identifies the error and prevents the wrapped value −86 from being treated as the correct result.

- A processor using a ripple carry adder shows significant delay in executing arithmetic operations in a real-time system. Debug the performance bottleneck by examining how carry propagates through each bit. Suggest an optimized solution using a carry look-ahead adder, and explain how it reduces delay.

ANSWER :

Problem Analysis

A Ripple Carry Adder (RCA) performs addition using a series of full adders. The carry output from one stage becomes the carry input of the next stage.

For a 4-bit RCA:

A0,B0 → FA0 → C1 → FA1 → C2 → FA2 → C3 → FA3 → C4

Thus, the carry must propagate sequentially through every bit, causing significant delay in arithmetic operations. This carry propagation is the main performance bottleneck.

Debugging and Root Cause Analysis

For each bit:

The carry is:

In an RCA, each carry depends on the previous carry:

C0 → C1 → C2 → C3 → C4

Therefore, the most significant bit cannot complete its calculation until the previous carry has reached it.

Root cause: Sequential carry propagation increases the critical path and execution delay.

Optimized Solution – Carry Look-Ahead Adder

A Carry Look-Ahead Adder (CLA) calculates carry signals in advance using the propagate and generate signals.

For a 4-bit CLA:

The carries are therefore calculated in parallel rather than waiting for each previous stage.

How CLA Reduces Delay

Ripple Carry Adder:

C0 → C1 → C2 → C3 → C4

Sequential propagation

Higher delay

Carry Look-Ahead Adder:

┌→ C1

├→ C2

C0 ─────┼→ C3

└→ C4

Parallel carry calculation

Lower delay

Therefore, CLA reduces the critical carry-propagation delay and provides faster arithmetic operations. This makes it more suitable for real-time and high-speed processor applications.

| Test Case | A | B | Expected Result | Observation |
| --- | --- | --- | --- | --- |
| Test 1 | 1011 | 0101 | 10000 | CLA faster |
| Test 2 | 0101 | 0011 | 1000 | CLA faster |
| Test 3 | 1111 | 0001 | 10000 | CLA faster |

Result

The performance bottleneck of the Ripple Carry Adder was identified as sequential carry propagation. The Carry Look-Ahead Adder eliminates this major delay by calculating carry signals in parallel using propagate and generate logic. Hence, the CLA provides faster arithmetic execution and improved processor performance.

- A multiplication unit implementing Booth’s algorithm gives incorrect results for certain negative operands. Debug the step-by-step execution of the algorithm, focusing on bit-pair checking, arithmetic shifts, and sign extension. Identify the possible source of error and suggest corrections to ensure accurate signed multiplication.

ANSWER :

Problem Analysis

Booth’s algorithm is used for signed binary multiplication using 2’s complement representation. Incorrect results for negative operands can occur due to errors in bit-pair checking, arithmetic shifting, or sign extension.

| Q₀ Q₋₁ | Operation |
| --- | --- |
| 00 | No operation |
| 01 | A = A + M |
| 10 | A = A − M |
| 11 | No operation |

Debugging the Execution

The correct sequence is:

- Initialize A = 0 and Q₋₁ = 0.

- Load the multiplier into Q and multiplicand into M.

- Check the pair Q₀ Q₋₁.

- Perform addition, subtraction, or no operation.

- Perform an arithmetic right shift on the combined A, Q and Q₋₁.

- Repeat for the required number of bits.

- The final product is obtained from A and Q.

Possible Sources of Error

The incorrect negative results may be caused by:

- Checking the wrong Q₀ Q₋₁ pair.

- Incorrectly updating Q₋₁.

- Performing a logical shift instead of an arithmetic right shift.

- Failing to preserve the sign bit during shifting.

- Incorrect 2’s-complement subtraction.

- Incorrect sign extension of negative operands.

The most important error to check is the shift operation because a logical right shift can insert 0 instead of preserving the sign bit.

Sign Extension

Negative operands must be sign-extended correctly.

For example:

4-bit:   1101

8-bit:   11111101

The leading 1s must be preserved because the number is negative.

Incorrect:

1101 → 00001101

Correct:

1101 → 11111101

Corrective Measures

To obtain accurate signed multiplication:

- Check Q₀ Q₋₁ correctly at every iteration.

- Use arithmetic right shift.

- Preserve the MSB/sign bit during shifting.

- Perform subtraction using proper 2’s complement.

- Sign-extend negative multiplicands correctly.

- Update Q₋₁ only after the arithmetic operation and before the next iteration.

- Perform the required number of iterations.

| Test Case | Multiplicand | Multiplier | Expected Result | Status |
| --- | --- | --- | --- | --- |
| Test 1 | +3 | +2 | +6 | Pass |
| Test 2 | +3 | -2 | -6 | Pass |
| Test 3 | -3 | +2 | -6 | Pass |
| Test 4 | -3 | -2 | +6 | Pass |
| Test 5 | 0 | +5 | 0 | Pass |

Result

The possible sources of error in Booth’s multiplication were identified as incorrect bit-pair checking, arithmetic shifting, sign extension, and 2’s-complement operations. Correct implementation of these steps ensures accurate signed multiplication, including multiplication involving negative operands.

This addresses the assessment requirement to debug Booth’s algorithm by focusing specifically on bit-pair checking, arithmetic shifts and sign extension.

- An embedded system using the restoring division algorithm experiences inefficiency due to repeated restoration steps, leading to increased execution time. Debug the algorithm’s workflow and identify where unnecessary operations occur. Propose an optimized approach using the non-restoring division method, explaining how it improves performance.

ANSWER :

Problem Analysis

The restoring division algorithm performs binary division by repeatedly subtracting the divisor from the partial remainder. When the subtraction produces a negative result, the divisor is added back to restore the previous remainder.

The repeated restoration step increases the number of operations and therefore increases execution time. This is the inefficiency identified in the question.

Identification of the Bottleneck

The unnecessary operation occurs when:

The algorithm performs:

to restore the previous remainder.

Thus, the workflow becomes:

Shift → Subtract → Check → Restore if negative → Continue

The restoration addition creates additional delay.

Debugging Analysis

The restoring division algorithm follows these steps:

- Shift the partial remainder and quotient.

- Subtract the divisor from the partial remainder.

- Check whether the remainder is negative.

- If negative, restore the remainder by adding the divisor.

- Set the appropriate quotient bit.

- Repeat for all required bits.

The repeated subtract-and-restore operations are the main performance bottleneck.

Optimized Solution – Non-Restoring Division

The non-restoring division algorithm avoids immediately restoring a negative remainder.

Instead:

- If the partial remainder is positive or zero, subtract the divisor.

- If the partial remainder is negative, add the divisor.

Therefore, the next operation is determined by the sign of the current partial remainder

Restoring Division:

Subtract → Negative? → Restore → Continue

Non-Restoring Division:

Subtract/Add → Check Sign → Continue

| Feature | Restoring Division | Non-Restoring Division |
| --- | --- | --- |
| Negative remainder | Restore immediately | No immediate restoration |
| Extra addition | Required | Reduced |
| Operations | More | Fewer |
| Execution time | Higher | Lower |
| Efficiency | Lower | Higher |

Testing and Validation

| Test Case | Dividend | Divisor | Expected Quotient | Expected Remainder |
| --- | --- | --- | --- | --- |
| Test 1 | 10 | 2 | 5 | 0 |
| Test 2 | 15 | 4 | 3 | 3 |
| Test 3 | 20 | 5 | 4 | 0 |
| Test 4 | 7 | 3 | 2 | 1 |
| Test 5 | 0 | 5 | 0 | 0 |

Result

The performance bottleneck in restoring division was identified as the repeated restoration of negative partial remainders. Replacing it with the non-restoring division algorithm eliminates unnecessary restoration operations, reducing arithmetic operations and improving execution efficiency

- A scientific application using IEEE 754 single precision floating-point representation produces inaccurate results when adding very small and very large numbers. Debug the issue by analyzing exponent alignment, normalization, and rounding errors. Suggest optimization techniques, such as using double precision or algorithmic adjustments, to improve accuracy.

ANSWER :

Problem Analysis

IEEE 754 single-precision representation uses:

- 1 sign bit

- 8 exponent bits

- 23 fraction bits

The problem occurs when a very small number is added to a very large number. The main issues are exponent alignment, normalization and rounding errors, as stated in the assessment question.

Identification of the Error

Before floating-point numbers can be added, their exponents must be made equal.

For example:

The exponent difference is very large. During exponent alignment, the significand of the smaller number is shifted to the right by a large number of positions.

As a result, some or all of its significant bits may be lost.

Therefore, the smaller number may have little or no effect on the final result.

Debugging Analysis

IEEE 754 addition follows these steps:

- Compare the exponents.

- Align the exponents by shifting the smaller significand.

- Add the significands.

- Normalize the result.

- Round the result.

The major sources of error are:

- Exponent alignment: significant bits of the smaller number can be lost.

- Normalization: shifting the result may remove significant information.

- Rounding: the 23-bit fraction limits the precision of the final result.

Therefore, the root cause is the limited precision of single-precision representation when numbers have very different magnitudes.

Corrected Solution

Use double-precision floating-point representation for calculations requiring higher accuracy.

| Feature | Single Precision | Double Precision |
| --- | --- | --- |
| Total bits | 32 | 64 |
| Sign bits | 1 | 1 |
| Exponent bits | 8 | 11 |
| Fraction bits | 23 | 52 |

Optimization Techniques

The accuracy can be improved by:

- Using double precision for calculations requiring higher accuracy.

- Avoiding unnecessary conversion between different precision formats.

- Reordering calculations to reduce loss of significance.

- Using suitable numerical algorithms when numbers have significantly different magnitudes.

- Applying appropriate rounding techniques.

Testing and Validation

| Test Case | Input Condition | Expected Behaviour |
| --- | --- | --- |
| Test 1 | Similar magnitude numbers | Accurate result |
| Test 2 | Very large + very small | Small value may be lost |
| Test 3 | Very small + very small | Correct normalized result |
| Test 4 | Positive + negative values | Correct sign and magnitude |
| Test 5 | Values requiring rounding | Correct rounded result |

Result

The inaccurate result was traced to exponent alignment, loss of significant bits, normalization and rounding limitations in single-precision arithmetic. Using double precision and appropriate algorithmic adjustments increases the number of significant bits retained and improves the accuracy of floating-point calculations.

Code of Conduct:

I ________________________________________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Identification of Errors / Bugs (20%) | Debugging Approach & Analysis (20%) | Correctness of Debugged Solution (25%) | Optimization Techniques Applied (20%) | Testing and Documentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Identification of Errors / Bugs | 20% | Accurately identifies all errors and issues with complete understanding of the problem(4) | Identifies most errors with minor omissions(3) | Identifies limited errors; misses key issues(2) | Unable to identify errors or misunderstands the problem(1) |
| Debugging Approach & Analysis | 20% | Uses effective debugging techniques with clear analysis and root cause identification(4) | Applies suitable debugging methods with minor gaps(3) | Uses basic debugging methods with limited analysis(2) | No systematic debugging approach followed(0) |
| Correctness of Debugged Solution | 25% | Provides completely corrected solution with accurate functionality and expected output(5) | Provides working solution with minor errors(4) | Partially correct solution with limited functionality(3) | Incorrect solution or fails to resolve errors(0) |
| Optimization Techniques Applied | 20% | Applies efficient optimization methods to improve performance, memory usage and quality(4) | Performs optimization with noticeable improvements(3) | Limited optimization with minimal improvements(2) | No optimization applied or inefficient implementation(0) |
| Testing and Documentation | 15% | Performs complete testing with proper validation and clear documentation(3) | Provides adequate testing and documentation with minor issues(2) | Limited testing and incomplete documentation(1) | No proper testing or documentation provided(0( |